QRAE3

Features and Benefits:

- Man Down Alarm with real-time remote wireless notification

- Easy maintenance with field replaceable sensors and pump

- Fully automated bump testing and calibration with AutoRAE 2

- Pumped or diffusion models available

- Large graphic display can rotate 180º

Specifications:

- Size: Pumped – 140 mm x 82mm x 42mm
        Diffused - 145 mm x 82 mm x 42mm

- Weight: Pumped – 12.9 oz (365 g)

Diffused - 14.5 oz (410 g)

- Temperature: −20°C to 50°C

- Sampling Pump: Built-in pump or diffusion

- Sensor Range: CO – 0 to 500 ppm

H2S – 0 to 100 ppm

O2 – 0 to 30.0% VOL

LEL – 0 to 100% LEL

SO2 – 0 to 20 ppm

HCN – 0 to 50 ppm

NH3 – 100 ppm

PH3 – 20 ppm

CL2 – 0 to 50 ppm

NO2 – 0 to 50 ppm

- Warranty: Full two-year warranty including sensors and battery

ToxiRAE Pro

Features and Benefits:

- Wireless access to real-time instrument readings and alarm status from any location

- Unmistakable five-way local and remote wireless notification of alarm conditions

- Intelligent sensors store calibration data, so they can be swapped in the field

- Most electrochemical sensors are shared with MultiRAE family instruments

- Largest display in its class

- Continuous datalogging

- Easy to use

- Reliable, rugged, and intrinsically safe

- Device Management with Honeywell SafetySuite

Specifications:

- Size: 4.6" H x 2.4" W x 1.2" D (118 x 60 x 30 mm)

- Weight: 7.76 oz. (220 g)

- Temperature Range:

- Sensor Range:

- NH3 – 0 to 100 ppm

- CO – 0 to 500 ppm

- CO (Ext. Range) – 0 to 2,000 ppm

- CO – H2-Comp. – 0 to 2,000 ppm

- CL2 – 0 to 50 ppm

- CIO2 – 0 to 1 ppm

- ETHYLENE OXIDE (ETO-A) – 0 to 100 ppm

- ETHYLENE OXIDE (ETO-B) – 0 to 10 ppm

- ETHYLENE OXIDE (ETO-C), EXT. RANGE – 0 to 500 ppm

- HCHO – 0 to 10 ppm

- H2 – 0 to 1,000 ppm

- HCL – 0 to 15 ppm

- HCN – 0 to 50 ppm

- H2S – 0 to 100 ppm

- H2S, EXT. RANGE – 0 to 1,000 ppm

- HF – 0 to 10 ppm

- CH3-SH – 0 to 10 ppm

- NO – 0 to 250 ppm

- N02 – 0 to 20 ppm

- O2 – 0 to 30% Vol

- PH3 - 0 to 20 ppm

- SO2 – 0 to 20 ppm

- Warranty: Two years on O2 sensor

Three years on H2S and CO sensors

Two years on non-consumable components

One year on all other sensors, battery, and other consumable parts

MultiRAE Lite

Features and Benefits:

- Wireless access to real-time instrument readings and alarm status from any location

- Unmistakable five-way local and remote wireless notification of alarm conditions including Man Down Alarm

- Over 25 interchangeable sensor options, including PID4 for VOCs, NDIR5 and catalytic for combustibles, and NDIR for CO2

- Intelligent sensors store calibration data, so they can be swapped in the field

- Large graphical display with easy-to-use, icon-driven user interface

- Continuous datalogging (6 months for 5 sensors, 24x7)

- Device Management with Honeywell SafetySuite

Specifications:

- Size: Pumped model: 7.6” H x 3.8” W x 2.6” D (193 x 96.5 x 66 mm)

Diffusion model: 6.9” x 3.8” x 2.2” (175 x 96.5 x 56 mm)

- Weight: Pumped model: 31 oz (880 g)

Diffusion model: 26.8 oz (760g)

- Temperature Range:

- Sensor Range:

- VOC 10.6 EV – 0 to 1,000 ppm

- CATALYTIC LEL – 0 to 100% LEL

- NDIR (0-100% LEL METHANE) – 0 to 100% Vol.

- NDIR (0-100% VOL. METHANE) – 0 to 100% Vol.

- CARBON DIOXIDE (CO2) NDIR – 0 to 50,000 ppm

- NH3 – 0 to 100 ppm

- CO – 0 to 500 ppm

- C0 (Ext. Range) – 0 to 2,000 ppm

- CO H2-COMP. – 0 to 2,000 ppm

- CO + - 0 to 500 ppm

- H2S COMBO – 0 to 200 ppm

- CL2 – 0 to 50 ppm

- CLO2 – 0 to 1 ppm

- ETHYLENE OXIDE (ETO-A) – 0 to 100 ppm

- ETHYLENE OXIDE (ETO-B) – 0 to 10 ppm

- ETHYLENE OXIDE (ETO-C), EXT. RANGE – 0 to 500 ppm

- FORMALDEHYDE (HCHO) – 0 to 10 ppm

- H2 – 0 to 1,000 ppm

- HCN – 0 to 50 ppm

- H2S – 0 to 100 ppm

- HYDROGEN SULFIDE (H2S), EXT. RANGE – 0 to 1,000 ppm

- METHYL MERCAPTAN (CH3-SH) – 0 to 10 ppm

- NITRIC OXIDE (NO) – 0 to 250 ppm

- NITROGEN DIOXIDE (NO2) – 0 to 20 ppm

- OXYGEN (O2) – 0 to 30% Vol.

- OXYGEN (LIQ O2) – 0 to 30% Vol.

- PHOSPHINE (PH3) – 0 to 20 ppm

- PHOSPHINE H (PH3 H) – 0 to 20 ppm

- SULFUR DIOXIDE (SO2) – 0 to 20 ppm