CHIYODA  ECONO DIAL SHIELD GAS SAVING REGULATOR WITH DIAL FLOWMETER

“ECONO-DIAL" is developed to minimize the discharge of wasteful gas in TIG, MIG, and CO2 welding.
It keeps gas flow at constant rate as from start of welding and assures smooth arc start and energy saving without causing deterioration of weld quality.
Approximately, 40% gas saving possible.

| MODEL | B-AR | B-CO2 | H-AR, HCO2 with mini-regulator | H-AR, HCO2 |
| --- | --- | --- | --- | --- |
| Applicable Gas | Argon, Mix Gas | CO2 | Argon, CO2, Mix gas | Argon, CO2, Mix gas |
| Connect To | To Cylinder | To Cylinder | To Pipeline Outlet | To Pipeline Outlet |
| Flow Rate Range | 3-15 NL/min. 6-30 NL/min. 10-50 NL/min. | 6-30 NL/min. 10-50 NL/min. | 3-15 NL/min. 6-30 NL/min. 10-50 NL/min. | 3-15 NL/min. 6-30 NL/min. 10-50 NL/min. |
| Inlet Pressure | Max. 165 kg/cm2 | Max. 60 kg/cm2 | 3.5-10 kg/cm2 | 3 kg/cm2 |
| Inlet Connection | J.I.s, C.G.A., B.S., D.I.N., etc |  | W22-M14 or others |  |
| Outlet Connection | 1/4" Hose Nipple |  |  |  |