Tokin Arc Model TKN-A1 Nozzle Cleaner is a robot peripheral cleaning device designed to maintain welding torch nozzles by removing spatter and debris efficiently. Its simple air-driven mechanism and interchangeable cleaning heads enable effective cleaning of both the nozzle tip and inner wall without damaging components, helping extend nozzle life and reduce downtime in welding operations.

- Pneumatic force turns the spring, allowing easy installation and safe operation.

- It stops turning if excessive force is applied to the head during nozzle cleaning. Thus, it minimizes damage to the nozzle.

- Numerous types of heads are available to accommodate nozzles.

- A “regulator and solenoid valve set” type is also available.

- A nozzle cleaner cover (spatter receptacle) is available as an option.

- Air Pressure : 0.2~0.5MPa

- Air Consumption : 60L/min

- Air Connection : Ø6 one-touch joint

- Outer Dimensions ( W x D x H ) : 200 x 110 x 190

- Weight : 1.5kgs.

The Tokin Arc Portable Butt Welder (PBW) is a compact and rugged portable welding unit engineered for field and workshop butt-welding applications. Designed to weld steel wire and small components with precision and safety, it delivers adjustable welding power suitable for various wire diameters while maintaining reliable performance and ease of use. Built with safety-compliant insulation and simple controls, the PBW is ideal for mobile welding tasks and on-site repairs where dependable, portable welding capability is essential.

- 230VAC, Single Phase 50/60Hz

- Electrical Power: 0.6kVA

- Applicable Wire Size: Ø0.8~2.0mm (Solid Wire only)

- Weight: 13.0 kgs.

The Wire Wizard Wire Sizer Model NSW-BW-WS-1 is a precision accessory for the Non-Stop® Wire Dispensing System, designed to prepare butt-welded wire ends for smooth feeding and reliable performance. It removes burrs from weld joints to ensure clean, consistent wire surfaces, eliminating snags and improving wire delivery in automated welding or high-volume production environments. This model is supplied without preset dies, allowing you to specify the correct die size for your wire application.

- For solid and flux cored wire

- Wire sizes up to .078” (2mm)

- Replacement dies available

EVO 2.2 – Mobile Filter Unit for Welding Smokes

The EVO 2.2 is a robust mobile filtering unit designed for efficient extraction and purification of welding fumes in workshops and industrial spaces where standard ventilation isn’t feasible. Built on a powder-coated steel frame with easy-roll castors, it delivers reliable, ready-to-use performance with a high-efficiency centrifugal fan and digital control panel. Its multi-stage filtration system achieves E12 class filtration efficiency (>99.5%), capturing particulate from processes like MIG, TIG, and MMA welding. Ideal for medium-low pollutant concentrations and dry welding fumes, the EVO 2.2 keeps the air cleaner and your workspace healthier.

Technical Features

| Model | EVO 2.2 |
| --- | --- |
| Fan capacity | 2500 m3/h |
| Extraction at hood | 1100 m3/h |
| Filtration efficiency | E12 >99.5 % |
| Filtering area | 14.5 m2 |
| Motor power | 1.1 – 230/50 kW – V/Hz |
| Noise | 73 db [A] |
| Dimension | 520 x 1010 x 920 h mm |
| Weight | 85 kgs. |

AER 201 – Portable Filtering Unit for Welding Smokes

The AER 201 is a compact, portable air filtration unit engineered for effective extraction and purification of welding fumes in enclosed or hard-to-reach areas where traditional ventilation isn’t practical. Designed for intermittent use with medium-to-low pollutant concentrations, it includes a flexible suction hose with capture nozzle and magnetic positioner for stable placement. Built with a sturdy, powder-coated steel body and a multi-stage filter system featuring a metallic spark arrestor, cartridge filter and activated carbon filter, the AER 201 delivers 99 % filtration efficiency to help maintain cleaner, healthier air during MIG, TIG, MMA and pulse welding operations.

Technical Features

| Model | AER 201 |
| --- | --- |
| Flexible Hose | Ø 50 – L. 2000 mm |
| Fan capacity | 250 m3/h |
| Extraction at hood | 110 m3/h |
| Filtration efficiency | M >99 % |
| Active carbons | 1.5 kgs. |
| Motor power | 1.1 – 230/50 kW – V/Hz |
| Noise | 80 db [A] |
| Dimension | 530 x 270 x 450 h mm |
| Weight | 20 kgs. |