The G.A.L. Gage Cat No. 1 Hi-Lo Welding Gauge is a multi-purpose inspection tool used primarily for pipe and weld fit-up, measuring in both standard (inches) and metric (millimeters) scales. It is made of stainless steel and helps to reduce radiographic rejects.

Key Specifications and Measurement Capabilities

The Cat No. 1 gauge can perform up to six different measurement functions:

- Internal Misalignment: Measures the internal mismatch of pipe wall alignment before and after tacking, which helps to cut down on radiographic rejects.

- Fit-up Gap: Measures the required gap after fit-up, specifically between 1/16" and 3/32".

- Material/Pipe Wall Thickness: Assesses the material or pipe wall thickness.

- Fillet Weld Leg Length: Checks the leg length of a fillet weld.

- Butt Weld Reinforcement (Crown Height): Measures the crown height or reinforcement of a butt weld.

- Bevel Angle Verification: Includes go/no-go features to verify a common beveled angle of 37-1/2 degrees.

Features

- Material: Premium stainless steel construction.

- Scale: Dual measurement system (inches on one side, millimeters on the other).

- Design: Slim, lightweight design (approx. 30% lighter than older models).

- Operation: Manual operation with an adjustable locking mechanism (locking screw) to hold measurements for reference.

- Compliance: Helps satisfy fit-up codes for organizations such as ASME, ANSI, API, and Military standards.

The G.A.L. Gage Cat No. 2 is a Single Purpose Hi-Lo Welding Gauge primarily used to measure internal misalignment (high-low) and root weld space in pipe or plate welding, checking fit-up gaps from 1/32" up to 3/16" (or 1/16" to 3/32" range for the economy model) with stainless steel construction for durability. It features both inch and metric scales, engraved markings for easy reading, and is available in economy (Cat #2) or professional (Cat #2 Pro) versions, with the Pro offering a specific tolerance of ±0.006" for accuracy in the 1/32" to 3/16" range.

Key Specifications & Features:

- Function: Measures internal misalignment (Hi-Lo) and root weld gap.

- Material: Stainless Steel, deburred for safety.

- Measurement Scale: Dual Inch/Metric.

- Range (Economy): Checks fit-up gaps (approx. 1/16" to 3/32").

- Range (Pro): For gaps between 1/32" and 3/16".

- Tolerance (Pro): ±0.006".

- Markings: Engraved for clarity.

- Versions: Economy (Cat #2) and Professional (Cat #2 Pro).

How it Works (for Fit-up):
You insert the gauge's alignment stops into the gap; if the thin part doesn't fit, the gap is smaller; if it goes through, it's larger, helping you determine if the fit-up meets code requirements before welding.

The G.A.L. Gage Cat No. 4 Bridge Cam Gauge is a versatile welding inspection tool used to measure various aspects of a weld in both inches and millimeters.

Key Specifications

| Feature | Specification |
| --- | --- |
| Material | Premium stainless steel construction. |
| Scales | Dual scales (inches/millimeters). |
| Design | Features one rotating dial and one sliding pointer for measurement. |
| Functionality | Can be used as a go/no-go gauge for quick assessment. |

Measurement Capabilities

The Cat No. 4 gauge can perform up to seven different types of measurements on welded surfaces and joints:

- Angle of preparation: Measures angles from 0° to 60°.

- Excess weld metal (capping size): Also known as weld reinforcement.

- Depth of undercut: Measures the depth of grooves next to the weld.

- Depth of pitting: Measures the depth of small depressions.

- Fillet weld throat dimension: Measures the distance from the joint root to the weld face.

- Fillet weld leg length: Measures the leg length of the weld.

- Misalignment (high-low): Measures the offset of plates or pipes.

The G.A.L. Gage Cat No. 6 (GAL-6) Automatic Weld Size AWS Type Gauge is a pocket-sized, stainless-steel tool used for the inspection and calibration of butt and fillet welds. It is designed for precise measurement of various weld features.

Key Specifications

- Manufacturer: G.A.L. Gage Company

- Part Number: GAL-6 (Inch version) or GAL-6-M (Metric version)

- Material: Stainless Steel

- Measurement Range: Up to 3/4" (0" - 0.75") for the inch version

- Dimensions: Approx. 3.5 x 3.5 x 0.6 inches (8.89 x 8.89 x 1.52 cm)

- Functions: Measures actual fillet weld leg length, permissible convexity, concavity, and weld reinforcement.

Measurement Functions

The gauge is used to check the following aspects of a weld:

- Fillet Weld Leg Size: Determines the actual leg length of the weld.

- Fillet Weld Throat Size: Checks the throat dimension against specifications.

- Weld Reinforcement: Measures the excess weld metal (capping size) on butt welds.

- Convexity & Concavity: Verifies the permissible tolerances for the profile of fillet welds.

- Undercut/Pits: Can check permissible tolerance of underfill.

The G.A.L. Gage Cat No. 8 is a 7-piece fillet weld gauge set used for the inspection of concave or convex fillet welds. It allows for fast and accurate measurement of eleven different fillet weld sizes, ranging from 1/8 inch to 1 inch (or metric equivalents).

Key Specifications

- Function: Measures fillet weld leg size and throat size to determine if they meet specifications.

- Measurement Range: 1/8", 3/16", 1/4", 5/16", 3/8", 7/16", 1/2", 5/8", 3/4", 7/8", and 1". Metric versions cover 3.175 mm to 25.4 mm.

- Material: Each blade is made from 1/4" x 4" cold-rolled stainless steel to resist rust and bending.

- Markings: All sizes and numerals are laser engraved into the surface to prevent rubbing off.

- Portability: The set of seven blades comes in a handy pocket case and weighs only about 4 ounces (0.26 lbs).

- Installation Method: Designed for use with arc-welding installation methods.